CHIPIT

DISCLAIMER:

THE AUTHOR MAKES NO REPRESENTATIONS ABOUT THE SUITABILITY OF THIS

SOFTWARE OR ABOUT ANY CONTENT OR INFORMATION MADE ACCESSIBLE BY THE SOFTWARE, FOR ANY PURPOSE.  THE SOFTWARE IS PROVIDED 'AS IS' WITHOUT EXPRESS OR IMPLIED WARRANTIES, INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE OR NONINFRINGEMENT.  THE ENTIRE RISK AS TO THE RESULTS AND PERFORMANCE OF THE SOFTWARE IS ASSUMED BY YOU.

IN NO EVENT WILL THE AUTHOR BE LIABLE TO YOU FOR ANY CONSEQUENTIAL, INCIDENTAL OR INDIRECT DAMAGES (INCLUDING DAMAGES FOR LOSS OF BYSINESS PROFITS, BUSINESS INTERRUPTION LOSS OF BUSINESS IMPORMATION, AND THE LIKE) ARISING OUT OF THE USE OR INABILITY TO USE THE SOFTWARE EVEN IF THE AUTHOR HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

###### FREEWARE

You are hereby licensed to make as many copies of this version of this software and documentation as you wish; give exact copies of the original version to anyone; and distribute this version of the software and documentation in its unmodified form via electronic means. There is no charge for any of the above.

Professional usage of that software is prohibited.

You are specifically prohibited from charging, or requesting donations, for any such copies, however made; and from distributing the software and/or documentation with other products (commercial or otherwise) without prior written permission.

This software is to be used for analysis, tests or Phone Book duplications only,

© Yves de Ryckel 2000

- Introduction

This software has been written for one main purpose : duplicating the Phone Book from one SIM card into another one. After a SIM card change, I was indeed very frustrated to have to retype about 50 names and numbers, using the GSM keypad only. Try it, you’ll enjoy that nice experience !

I’ve started to write a simple HEX terminal in DOS just to learn more about the SmartCard operating system..Then I’ve decided to write that application for Windows.

I’ve started with the simple HEX terminal and gradually added functions and buttons in order to provide the user with more flexibility and ease.

WARNING :

Please note that you might block your SIM card if you don’t know what you’re doing especially by sending commands via the terminal screen or making multiple wrong PIN entries. If you don’t have the PUK’s (Pin unblocking) codes, be extremely cautious !

- Requirements

Windows 32 bits (95, 98) . Not tested on NT nor on WIN2000

PC with  a Standard COM port

This program has been tested on an AMD K6 2 333 and on a Pentium 200 MMX

with 64 Mb RAM. Best display view in 800X600 .

One SIMcard/SmartCard simple interface

That program has been thoroughly  tested on the UNIPROG interface.

The description, Printed Board drawings and schematics are available on http://www.ccc.de

I’m pretty sure that this program could run also on a Maki interface as I’ve tested one of their programs on my UNIPROG. (Use the inverted CTS/DTR option in ChipIt.)

ChipIt should work also on other interfaces like the DumbMouse or SmartMouse compatible, but I cannot guarantee that because I don’t have that type of hardware.

Any feedback would be welcome.

One old SIMcard with a known PIN1/CHV1 code for trials.

- Installation.

All files have to be installed in the same directory. You need to install also to install :

VB40032.DLL        in your \Windows\System directory

COMDLG32.OCX  in your \Windows\System directory

MSCOMM32.OCX in your \Windows\System directory.

TabCtl32.OCX       in your \Windows\System directory

(Please note that the program can run if those files are left in the ChipIt directory.)

Though you may change the INI settings by using a Text Editor, do not modify the structure of the CHIPIT.INI file.

- ChipIt features

- WIN95/98 32 bits program

- Hexadecimal Terminal with no extra key to use when  entering commands

- Quick commands with buttons 'Select, Get Response and quick Dir EF’s...

- SIM File SELECTion through mnemonics or address

- Translation of ANSWERS (SW1-SW2)

- Customizable COM settings

- Dual polarity RESET (ATR) on CTS or DTR

- Phone Book Editing screen

- Compatible with the Uniprog interface and likely with other interfaces like Maki,

DumbMouse ...

- Session Logging in a text file

- Phone Book saved in delimited format

-     Automatic Parity detection (Even or Odd) upon Reset (Normal/Inverse)

The main screen is divided in 4 main sections :

- The Menu (Com parameters, Log file …)

- The Information (Responses) on the top part

- The Hexadecimal Terminal

- The Controls

In addition there are 3 tab’s :

- One with the Main screen (see above)

- Card Information (SST, ICC and IMSI)

- RFU (Reserved for future use)

4.1 -  Menu

4.1.1 – File

[Open LOG] file to open a file for the collection of all information from the Hex

Terminal screen.

[Close LOG] file to close the file and save it on your Hard Disk.

[Exit] to leave the application and return to Windows

4.1.2 – ComPort

This menu item gives access to PARAMETERS for the COM port settings, i.e Baud rate, word Length, Stop bits, Parity, Handshaking and Echo.

Once changed, those parameters are saved in the INI file.

4.1.3 – About

Displays the information on the ChipIT version

4.1.4 – Status

This is a label that provides the user with some information on RS232 control line status.(i.e. CD Error if you remove the card from the UNIPROG interface)

Please note that you may sometimes receive a TX Buffer Overflow error when

changing cards. Just acknowledge the Message Box in order to clear the TX buffer.

- Information

The Response to an MF (Master File) or DF (Data File) Selection is displayed

Here :

Sub DF : number of DF under the current directory

Sub EF : number of EF under the current directory

#KEYS : number of keys being used (PIN1, PIN2 ….)

CLKS   : Clock allowed Yes or No

PIN1    : number of trials left before PIN1 is blocked

PIN2	  : number of trials left before PIN2 is blocked

PUK1	  : number of trials left before PUK1 is blocked

PUK2   : number of trials left before PUK2 is blocked

Under DEFINED, you’ll find the status for each key showing whether or not the

key has been initialized. Not to be confused with the Enabled status of

PIN1.

6 - The Terminal screen

Using that terminal, you may enter all GSM 11.11 standard commands and enter in dialogue with the SIM card under test.

Prior to talk with the card, you must click on [ATR on CTS] or on [ATR on DTR].

You should get the Answer To Reset from the SIM card. If you don’t get any hex string on the Terminal screen, try to select the INV mode in order to invert the RESET

signal sent to the interface. (Maki™  interface with standard settings)

The UNIPROG interface uses the [ATR on CTS] signal.

Once you’ve got the ATR from the SIM, you may talk with the card.

Example :

Type 		: A0A4000002 [Enter]

Answer	: A4  (means that SIM has recognized the instruction code SELECT

Type 		: 7F20 [Enter]  (GSM Directory)

Answer	: 9Fxx (xx= 1A or 1B …) OK x Bytes

The responses to commands are translated according to the GSM 11.11 specification (SW1 and SW2) and are displayed in the Terminal window.

You may execute commands like those included in the annex 1… and even others from other cards in the direct convention. (see GSM 11.11 rev 6.2 for additional details)

All data’s displayed in that window may be logged in a file for further analysis . Prior to initiate a file logging, click on [CLS] to clear the hexadecimal terminal window if necessary and click on  [Open Log] file in the Controls window.

7 – CONTROLS

The bottom portion of the screen regroups all controls :

[ATR on CTS] 	: Reset the card using the CTS line. The polarity of that RESET

signal can be changed using the INV/NOR buttons

[ATR on DTR] 	: Reset the card using the DTR line. The polarity of that RESET

signal can be changed using the INV/NOR buttons

[ComPort] 		: enter in the COM: port settings menu

[INV] [NOR]		: RESET from LOW to HIGH or HIGH to LOW transition

[INV Toggle]		: RFU and blocked

[CLS Term]			: clear the Hex Terminal windows and reset the cursor position .

[Open LOG]		: duplicated in the File menu. Used to open a text file for data

logging.

[Close LOG]		: duplicated in the File menu. Used to close the text file.

The Log file is automatically closed upon exit.

[PIN1] … [PUK2] 	: Used to activate the respective key’s

[SELECT FILE]	: Used to SELECT a file in the SIM card.

You may use either the address or the mnemonic (Upper case   or  Lower Case.)

Example : to SELECT the MF you may enter MF or 3F00

All SELECT commands are starting from the root file MF, so

You may directly enter ADN (6F3A) for example to get access to

the Phone Book which is located under the TELECOM:7F10 DF

file. All recognized mnemonics are shown in the annex 2.

[GET RESPONSE] 	: Displays the parameters of a file after its SELECTion.

There are two different areas depending upon the file type

Selected, EF or DF/MF

- The Response to an MF (Master File) or DF (Data File) Selection gives the following information (see also above):

Sub DF : number of DF under the current directory

Sub EF : number of EF under the current directory

#KEYS : number of keys being used (PIN1, PIN2 ….)

CLKS   : Clock allowed Yes or No

PIN1    : number of trials left before PIN1 is blocked

PIN2	  : number of trials left before PIN2 is blocked

PUK1	  : number of trials left before PUK1 is blocked

PUK2   : number of trials left before PUK2 is blocked

Under DEFINED, you’ll find the status for each key showing whether or not the

key has been initialized. Not to be confused with the Enabled status of

PIN1.

- The Response to an EF (Elementary File)  Selection gives the following information :

File size 		: can be expressed in bytes for a transparent file or in

records for a linear file

File ID   		:  the standard identifier for the file (Address) after the

Back Slash

File Type 		: usually EF

INCREASE Status 	: only for cyclic files otherwise N/A

Security Access Conditions for that particular file i.e. READ/SEEK, UPDATE,

INCREASE, REHABILITATE and INVALIDATE

File status 		: may be VALIDATED or INVALIDATED

Structure of EF 	: Linear Fixed, Transparent or Cyclic

Length of a record 	: 00 in case of a Transparent file

[DIR EF’s]		: List  directly all EF’s under the MF or a SELECTed  DF

SELECT first a DF/MF and then click on DIR EF’s

Note : this feature is not documented in the GSM11.11 nor in

the ISO7816 [4]. It’s a combination of two commands that

gives directly the DF/MF structure.

It may not work with all SIM cards!

Please give me some feedback as I’ve tested that feature

only on local SIM cards.

[READ BINARY] 	: Read all bytes from a transparent file.  Prior to use that

command you must issue a SELECT command.

[READ RECORDS] : Read all records from a linear file.  Prior to use that

command you must issue a SELECT command.

## 7.1 - The PHONE BOOK group

Prior to use the Phone Book buttons, you MUST validate the PIN1/CHV1.

[PB Read] 	: reads the Phone Book data’s from the SIM card. Data’s are kept in

memory in an array. The Record # in process is displayed in the Rec #

in Process box. During the Read Process, dot’s are being sent to the

Terminal window in order to show the loading progress.

[PB Write]	: writes Phone Book Data’s into the SIM Card. That command can be

used to copy data’s coming out of a file or from another SIM card. If

you’ve change the card in the interface, don’t forget to Reset the card

and to enter a new valid PIN1 key prior to write Phone Book data’s. All

data’s are kept in memory. The access to the button is blocked If no

valid Phone Book data’s are available.

The Record # in process is displayed in the Rec #   in Process box.

During the Write Process, dot’s are being sent to the Terminal window

in order to show the loading progress.

If the original file to be uploaded has more records than the SIM capacity, then the file is truncated.

Sequences

Commands can be sequenced as follows :

To copy from SIM to SIM		[RESET]

[PIN1] (except of PIN1 disabled)

[P.B. READ]

[P.B. EDIT]  optional

change SIM card

[RESET]

[PIN1]

[P.B. WRITE]

To copy from File to SIM		[RESET]

[PIN1]

[P.B. LOAD]

[P.B. EDIT]  optional

[P.B. WRITE]

To copy from SIM to File		[RESET]

[PIN1]

[P.B. READ]

[P.B. EDIT]  optional

[P.B. SAVE]

[P.B. EDIT] 	: That button opens a simple editor in order to allow for changes or

updates. Records are displayed in groups  of 10.

### You may change the individual data’s and store the updated data’s

with the ENTER key. The ENTER key moves the cursor to the next

field AND stores the updated data’s in memory.

CAUTION : the TAB key only moves the cursor to the next field. So

if you change some data’s and use TAB to move to the

next field, you are going to loose the updates. Data’s will

remain unchanged !

The TON/NPI value is automatically recalculated during the editing

process.

The Name textbox width is automatically adjusted according to the

Maximum length allowed by the SIM format.

[P.B.SAVE] 	: Save the entire file on disk. Please that the saved format is different

from the SIM format. (delimited format)

The Format of a Phone Rec in .PHB file is as follows

NAMEonXChar/XX/(+)####### ....

!   !-> data separators /

!

!-> TON/NPI (= FF if no valid data's)

The data’s could be imported in a spreadsheet using the field separators “/”. I the name has less than the maximum allowed X characters, then Chipit adds hex FF (255 dec) up to the maximum length.

The + sign for International code is skipped when the record is

converted into SIM format.

Note: 	You must provide with the correct TON/NPI number if you edit the

data’s in a spreadsheet or a text editor !

There is no provision for numbers with more then 20 digits.

Note : I was unable to find the GSM 04.08 [14] specification for the TON/NPI

codification.

However I’ve found a spec for TON/NPI for PBX’s and I’ve guessed that it

could match the GSM needs.

My assumptions :

Ton/NPI = "91"  ' international code with conditional + for 00

Ton/NPI = "80"  ' unknown or national

##### Ton/NPI = "81"  ' extended international number

##### Ton/NPI = "81"  ' ex. short codes

If you find the complete information, please let me know, so that I can program the correct specification.

[P.B. LOAD]	: Load a Phone Book file from disk with the default suffix .PHB

[QUIT ChipIT]: Close the files and the COM port and exit to the Operating System

- Bugs and suggestions

Please report any bug or suggestion to yderyckel@yahoo.fr .or

Yves.de.ryckel@euronet.be

I’ll do my best to correct to bug’s asap and to incorporate any suggestion.

Please report if you encountered timing problems with very fast computers.

If yes, I’m going to add timing settings (delay ms) in the INI file

Please note that I’m programming on part time only. So I’m going to add other functions only when I’ve enough time…

Yves de Ryckel  - Belgium

Doc file for ChipIT 2.2.0

Annex 1  - INStructions

| Value | Command name |  |
| --- | --- | --- |
| “F2” | STATUS |  |
| '20' | VERIFY |  |
| 'A2' | SEEK |  |
| '32' | INCREASE |  |
| “24” | CHANGE CHV |  |
| '20' | VERIFY CHV |  |
| “28” | ENABLE CHV |  |
| “26” | DISABLE CHV |  |
| “2C” | UNBLOCK CHV |  |
| “04” | INVALIDATE |  |
| “08” | ??? |  |
| “B4” | GET DIR ??? (in Response) |  |
| “B6” | ??? |  |
| “B7” | ??? |  |
| “B8” | ??? |  |
| “44” | REHABILITATE |  |
| ' 88' | INTERNAL AUTHENTICATE |  |
| “FA” | SLEEP |  |
| “10” | TERMINAL PROFILE |  |
| “C2” | ENVELOPE |  |
| “12” | FETCH |  |
| “14” | TERMINAL RESPONSE |  |
| 'A4' | SELECT FILE |  |
| 'B0' | READ BINARY |  |
| 'B2' | READ RECORD(S) |  |
| 'C0' | GET RESPONSE |  |
| 'C2' | ENVELOPE |  |
| 'CA' | GET DATA |  |
| 'D0' | WRITE BINARY |  |
| 'D2' | WRITE RECORD |  |
| 'D6' | UPDATE BINARY |  |
| 'DA' | PUT DATA |  |
| 'DC' | UPDATE DATA |  |
| 'E2' | APPEND RECORD |  |

## Annex 2 : Mnemonics

"MF", "3F00"				SIM Current File = "MF-3F00-MF "

"CHV1", "0000", "PIN1"		SIM Current File = "MF-3F00-CHV1 "

"ICC", "0002"			SIM Current File = "MF-3F00-ICC "

"0011", "KeyExt", "Key Ext"	SIM Current File = "MF-3F00-KeyExt "

"CHV2", "0100", "PIN2"		SIM Current File = "MF-3F00-CHV2 "

"1001"					SIM Current File = "MF-3F00-???? "

"7F21"				SIM Current File = "DF-7F21 "

"GSM", "7F20"			SIM Current File = "DF-7F20-GSM"

"TELECOM", "7F10"		SIM Current File = "DF-7F10-TELECOM"

"IS-41", "7F22"			SIM Current File = "DF-7F22-IS41"

"IS41"					SIM Current File = "DF-7F22-IS41"

"ICCID", "2FE2"			SIM Current File = "EF-2FE2-ICCID"

"ELP", "2F05"			SIM Current File = "EF-2F05-ELP"

"IRIDIUM", "5F30"			SIM Current File = "DF-5F30-IRIDIUM"

"GLOBST", "5F31"			SIM Current File = "DF-5F31-GOBST"

"ICO", "5F32"			SIM Current File = "DF-5F32-ICO"

"ACES", "5F33"			SIM Current File = "DF-5F33-ACES"

"PCS1900", "5F40"			SIM Current File = "DF-5F40-PCS1900"

"LP", "6F05"				SIM Current File = "EF-6F05-LP"

"IMSI", "6F07"			SIM Current File = "EF-6F07-IMSI"

"KC", "6F20"				SIM Current File = "EF-6F20-Kc"

"PLMNSEL", "6F30"		SIM Current File = "EF-6F30-PLMNSEL"

"HPLMN", "6F31"			SIM Current File = "EF-6F31-HPLMN"

"ACMMAX", "6F37"			SIM Current File = "EF-6F37-ACMMAX"

"SST", "6F38"			SIM Current File = "EF-6F38-SST"

"ACM", "6F39"			SIM Current File = "EF-6F39-ACM"

"GID1", "6F3E"			SIM Current File = "EF-6F3E-GID1"

"GID2", "6F3F"			SIM Current File = "EF-6F3F-GID2"

"PUCT", "6F41"			SIM Current File = "EF-6F41-PUCT"

"CBMI", "6F45"			SIM Current File = "EF-6F45-CBMI"

"SPN", "6F46"			SIM Current File = "EF-6F46-SPN"

"CBMID", "6F48"			SIM Current File = "EF-6F48-CBMID"

"BCCH", "6F74"			SIM Current File = "EF-6F74-BCCH"

"ACC", "6F78"			SIM Current File = "EF-6F78-ACC"

"FPLMN", "6F7B"			SIM Current File = "EF-6F7B-FPLMN"

"LOCI", "6F7E"			SIM Current File = "EF-6F7E-LOCI"

"AD", "6FAD"			SIM Current File = "EF-6FAD-AD"

"PHASE", "6FAE"			SIM Current File = "EF-6FAE-PHASE"

"VGCS", "6FB1"			SIM Current File = "EF-6FB1-VGCS"

"VGCSS", "6FB2"			SIM Current File = "EF-6FB2-VGCSS"

VBS", "6FB3"			SIM Current File = "EF-6FB3-VBS"

"VBSS", "6FB4"			SIM Current File = "EF-6FB4-VBSS"

"EMLPP", "6FB5"			SIM Current File = "EF-6FB5-eMLPP"

"AAEM", "6FB6"			SIM Current File = "EF-6FB6-AAeM"

"ECC", "6FB7"			SIM Current File = "EF-6FB7-ECC"

"CBMIR", "6F50"			SIM Current File = "EF-6F50-CBMIR"

"NIA", "6F51"			SIM Current File = "EF-6F51-NIA"

"KCGPRS", "6F52"			SIM Current File = "EF-6F52-KcPRS"

"LOCIGPRS", "6F53"		SIM Current File = "EF-6F53-LOCIGPRS"

"ADN", "6F3A"			SIM Current File = "EF-6F3A-ADN"

"FDN", "6F3B"			SIM Current File = "EF-6F3B-FDN"

"SMS", "6F3C"			SIM Current File = "EF-6F3C-SMS"

"CCP", "6F3D"			SIM Current File = "EF-6F3D-CCP"

"MSISDN", "6F40"			SIM Current File = "EF-6F40-MSISDN"

"SMSP", "6F42"			SIM Current File = "EF-6F42-SMSP"

"SMSS", "6F43"			SIM Current File = "EF-6F43-SMSS"

"LND", "6F44"			SIM Current File = "EF-6F44-LND"

"SMSR", "6F47"			SIM Current File = "EF-6F47-SMSR"

"SDN", "6F3A"			SIM Current File = "EF-6F3A-SDN"

"EXT1", "6F4A"			SIM Current File = "EF-6F4A-EXT1"

"EXT2", "6F4B"			SIM Current File = "EF-6F4B-EXT2"

"EXT3", "6F4C"			SIM Current File = "EF-6F4C-EXT3"

"BDN", "6F4D"			SIM Current File = "EF-6F4D-BDN"

"EXT4", "6F4E"			SIM Current File = "EF-6F4E-EXT4"

"2FE4"				SIM Current File = "EF-2FE4-????"

"2FE5"				SIM Current File = "EF-2FE5-????"

Or any 4 digits address.

Please note that  DF’s like IRIDIUM etc.. may not be implemented in your card.

Command List

[Seek]

Type=0

Instruction=A0 A2 xOffset(1Byte) xMode(1Byte) xLength(1Byte) xData

Response Length=0

Response Time=5000

Error1=62 81, Data may be corrupted

Error2=98 10, The file at the current pointer is invalidated

Error3=94 00, DF or Cyclic EF is selected

Error4=6A 80, Pattern not found/current file type is inconsistent with instruction

Error5=6B 00, Offset out of the EF / incorrect mode

[Increase]

Type=0

Instruction=A0 32 00 00 03 xdata(3Bytes)

Response Length=0

Response Time=5000

Error1=65 00, Too much Data for PRO mode

Error2=65 81, Update impossible

Error3=94 00, No EF selected

Error4=6A 80, Current file type inconsistent with the instruction

Error5=98 50, Increase can not be performed Max value reached

[Decrease]

Type=0

Instruction=A0 30 00 00 03 xdata(3Bytes)

Response Length=0

Response Time=5000

Error1=65 00, Too much Data for PRO mode

Error2=65 81, Update impossible

Error3=94 00, No EF selected

Error4=94 08, Current file type inconsistent with the instruction

Error5=98 50, Decrement can not be performed Min value reached

[IncreaseStamped]

Type=0

Instruction=A0 36 00 00 03 xdata(3Bytes)

Response Length=0

Response Time=5000

Error1=65 00, Too much Data for PRO mode

Error2=65 81, Update impossible

Error3=69 85, No Give Challenge performed before

Error4=94 00, No EF selected

Error5=98 50, Increase can not be performed Max value reached

[DecreaseStamped]

Type=0

Instruction=A0 34 00 00 03 xdata(3Bytes)

Response Length=0

Response Time=5000

Error1=65 00, Too much Data for PRO mode

Error2=65 81, Update impossible

Error3=69 85, No Give Challenge performed before

Error4=94 00, No EF selected

Error5=98 50, Decrease can not be performed Min value reached

[ExternalAuthentication]

Type=0

Instruction=A0 82 00 00 07 xKeyNumber(1Byte) zKey(8Bytes)

Response Length=0

Response Time=6000

Error1=98 40, Key blocked

Error2=63 00, Wrong Cryptogram

Error3=69 85, No get challenge before

[InternalAuthentication]

Type=0

Instruction=A0 88 00 xKeyNumber(1Byte) 08 zKey(8Bytes)

Response Length=0

Response Time=6000

[Invalidate]

Type=0

Instruction=A0 04 00 00 00

Response Length=0

Response Time=6000

Error1=98 10, The file at the current pointer is invalidated

Error2=98 02, No CHV or Key defined

Error3=94 00, No EF selected

[Rehabilitate]

Type=0

Instruction=A0 44 00 00 00

Response Length=0

Response Time=6000

Error1=98 10, The file at the current pointer is not invalidated

Error2=94 00, No EF selected

[Verify]

Type=0

Instruction=A0 2A 00 xKeyNumber(1Byte) 08 zKey(8Bytes)

Response Length=0

Response Time=4000

Error1=98 04, Key verify rejected

Error2=98 40, Key blocked

Error3=98 02, No Key defined

Comment1=SC length can vary.   For now fixed @ 08 for compatibility

[VerifyCHV]

Type=0

Instruction=A0 20 00 01 08 zPIN(8Bytes)

Response Length=0

Response Time=4000

Error1=98 04, CHV verify rejected

Error2=98 40, CHV blocked

Error3=98 02, No CHV defined

Comment1=SC length can vary.   For now fixed @ 08 for compatibility

[UnblockCHV]

Type=0

Instruction=A0 2C 00 01 10 zUnblockPin(8Bytes) zNewPIN(8Bytes)

Response Length=0

Response Time=6000

Error1=98 04, Unblocking CHV verification rejected

Error2=98 40, Unblocking CHV is blocked

Error3=98 02, No CHV defined

[ChangeCHV]

Type=0

Instruction=A0 24 00 01 10 zOldPIN(8Bytes) zNewPIN(8Bytes)

Response Length=0

Response Time=5000

Comment=Changes secret code

[DeleteFile]

Type=0

Instruction=A0 E4 00 00 02 xFileID(2Bytes)

Response Length=0

Response Time=5000

Error1=94 04, File ID not found

[GetResponse]

Type=0

Instruction=A0 C0 00 00 xResponseBytes(1Byte)

Response Length=250

Response Time=5000

Comment=Returns status of selected file.

[CreateFile]

Type=0

Instruction=A0 E0 xP1(1Byte) xP2(1Byte) xP3(1Byte) FF FF XBlockSize(2Bytes) xFileID(2Bytes) XFileType(1Bytes) xValidAction(1Byte) XAccessCond(3Bytes) xStatus(1Byte) xLengthofFollowingData(1Byte) XAccessKeys(3Bytes) XRLength(1Byte)

Response Length=0

Response Time=5000

Error1=94 08, Record Length too long or ID already exists

Error2=6A 84, Insufficient memory space available for creating file

Comment1=AC and KN are normal 2 bytes at expense of excluding "rehabilitate" and "invalidate"

[CreateDirectory]

Type=0

Instruction=A0 E0 00 FF 10 FF FF xBlockSize(2Bytes) xFileID(2Bytes) 38 xValidAction(1Byte) xAccessCond(3Bytes) xStatus(1Byte) 03 xAccessKeys(3Bytes)

Response Length=0

Response Time=5000

Error1=6A 80, File ID already existing in the parent DF

Error2=98 10, Contradiction wth invalidation status

Comment=Review (1) 3rd byte is Init Type, (2) AC coding, (3) Last 6 bytes

[CreateLinearFixed]

Type=0

Instruction=A0 E0 00 xKeysRecs(1Byte) 11 FF FF XBlockSize(2Bytes) xFileID(2Bytes) 02 xValidAction(1Byte) XAccessCond(3Bytes) xStatus(1Byte) 04 XAccessKeys(3Bytes) XRLength(1Byte)

Response Length=0

Response Time=5000

Error1=6A 80, Record Length too long or ID already exists

Error2=98 10, Contradiction with invalidation status

Error3=98 02, No CHV or KEY defined

[CreateLinearVariable]

Type=0

Instruction=A0 E0 00 FF 10 FF FF XBlockSize(2Bytes) xFileID(2Bytes) 04 xValidAction(1Byte) XAccessCond(3Bytes) xStatus(1Byte) 03 XAccessKeys(3Bytes)

Response Length=0

Response Time=5000

Error1=6A 80, Record Length too long or ID already exists

Error2=98 10, Contradiction with invalidation status

Error3=98 02, No CHV or KEY defined

[CreateCyclic]

Type=0

Instruction=A0 E0 00 xKeysRecs(1Byte) 11 FF FF XBlockSize(2Bytes) xFileID(2Bytes) 06 xValidAction(1Byte) XAccessCond(3Bytes) xStatus(1Byte) 04 XAccessKeys(3Bytes) xRLength(1Byte)

Response Length=0

Response Time=5000

Error1=6A 80, Record Length too long or ID already exists

Error2=98 10, Contradiction with invalidation status

Error3=98 02, No CHV or KEY defined

[CreateCalculation]

Type=0

Instruction=A0 E0 00 FF 10 FF FF XBlockSize(2Bytes) 00 01 01 xValidAction(1Byte) XAccessCond(3Bytes) xStatus(1Byte) 03 XAccessKeys(3Bytes)

Response Length=0

Response Time=5000

Error1=6A 80, Record Length too long or ID already exists

Error2=98 10, Contradiction with invalidation status

Error3=98 02, No CHV or KEY defined

[CreateTransparent]

Type=0

Instruction=A0 E0 00 FF 10 FF FF XBlockSize(2Bytes) xFileID(2Bytes) 01 xValidAction(1Byte) XAccessCond(3Bytes) xStatus(1Byte) 03 XAccessKeys(3Bytes)

Response Length=0

Response Time=5000

Error1=6A 80, Record Length too long or ID already exists

Error2=98 10, Contradiction with invalidation status

Error3=98 02, No CHV or KEY defined

[CreateVerification]

Type=0

Instruction=A0 E0 00 FF 10 FF FF XBlockSize(2Bytes) 00 11 01 xValidAction(1Byte) XAccessCond(3Bytes) xStatus(1Byte) 03 XAccessKeys(3Bytes)

Response Length=0

Response Time=5000

Error1=6A 80, Record Length too long or ID already exists

Error2=98 10, Contradiction with invalidation status

Error3=98 02, No CHV or KEY defined

[CreatePIN]

Type=0

Instruction=A0 E0 00 FF 10 FF FF 00 17 00 00 01 xValidAction(1Byte) XAccessCond(3Bytes) xStatus(1Byte) 03 XAccessKeys(3Bytes)

Response Length=0

Response Time=5000

Error1=6A 80, Record Length too long or ID already exists

Error2=98 10, Contradiction with invalidation status

Error3=98 02, No CHV or KEY defined

[Select]

Type=0

Instruction=A0 A4 00 00 02 xFileID(2Bytes)

Response Length=0

Response Time=4000

Error1=94 04, File ID not found

Error2=62 81, Data may be corrupted

Comment=Also know as "Select"

[ReadRecord]

Type=0

Instruction=A0 B2 xRecNum(1Byte) xMode(1Byte) XLengthofFollowingData(1Byte)

Response Length=OpenAll

Response Time=4000

Error1=62 81, Record is corrupted

Error2=94 00, No EF selected as current

Error3=94 02, Out of Range

[ReadBinary]

Type=0

Instruction=A0 B0 xOffsetHigh(1Byte) xOffsetLow(1Byte) Xlength(1Byte)

Response Length=OpenAll

Response Time=5000

Error1=94 00, No EF selected as current

[CreateRecord]

Type=0

Instruction=A0 E2 00 00 xLengthofFollowingData(1Byte) zData

Response Length=0

Response Time=4000

Error1=94 02, Out of Range

Error2=6A 84, Insufficient Memory

Error3=98 10, File At The Current Pointer Is Invalidated

Comment=Documentation refers to this as "Create Record"

[UpdateRecord]

Type=0

Instruction=A0 DC xRecNum(1Byte) xMode(1Byte) xLengthofFollowinData(1Byte) zData

Response Length=0

Response Time=4000

Error1=94 02, Out of Range

Error2=65 81, Update Impossible

[UpdateBinary]

Type=0

Instruction=A0 D6 xOffsetHigh(1Byte) xOffsetLow(1Byte) XLengthofFollowingData(1Byte) zData

Response Length=0

Response Time=7000

Error1=98 10, File at the current pointer is invalidated

Error2=65 81, Update Impossible

Error3=6B 00, Offset out of the EF