AVR part name:      (141 parts currently listed)

# Feature configuration

This allows easy configuration of your AVR device. All changes will be applied instantly.

Features

Clock output on PORTB0; [CKOUT=0]

Divide clock by 8 internally; [CKDIV8=0]

Boot Reset vector Enabled (default address=$0000); [BOOTRST=0]

Preserve EEPROM memory through the Chip Erase cycle; [EESAVE=0]

Watch-dog Timer always on; [WDTON=0]

Serial program downloading (SPI) enabled; [SPIEN=0]

Debug Wire enable; [DWEN=0]

Reset Disabled (Enable PC6 as i/o pin); [RSTDISBL=0]

# Manual fuse bits configuration

This table allows reviewing and direct editing of the AVR fuse bits. All changes will be applied instantly.
Note:  means unprogrammed (1);  means programmed (0).

| Bit | Low | High | Extended |
| --- | --- | --- | --- |
| 7 | CKDIV8 Divide clock by 8 | RSTDISBL External reset disable |  |
| 6 | CKOUT Clock output | DWEN debugWIRE Enable |  |
| 5 | SUT1 Select start-up time | SPIEN Enable Serial programming and Data Downloading |  |
| 4 | SUT0 Select start-up time | WDTON Watchdog Timer Always On |  |
| 3 | CKSEL3 Select Clock Source | EESAVE EEPROM memory is preserved through chip erase |  |
| 2 | CKSEL2 Select Clock Source | BOOTSZ1 Select boot size | BODLEVEL2 Brown-out Detector trigger level |
| 1 | CKSEL1 Select Clock Source | BOOTSZ0 Select boot size | BODLEVEL1 Brown-out Detector trigger level |
| 0 | CKSEL0 Select Clock Source | BOOTRST Select reset vector | BODLEVEL0 Brown-out Detector trigger level |

# Current settings

These fields show the actual hexadecimal representation of the fuse settings from above. These are the values you have to program into your AVR device. Optionally, you may fill in the numerical values yourself to preset the configuration to these values. Changes in the value fields are applied instantly (taking away the focus)!

| Low | High | Extended | Action | AVRDUDE arguments |
| --- | --- | --- | --- | --- |
| 0x | 0x | 0x* | Apply manual changes to the values on the left side, or load factory default values for the selected device. | -U lfuse:w:0x62:m -U hfuse:w:0xd9:m -U efuse:w:0xff:m Select (try triple-click) and copy-and-paste this option string into your avrdude command line. You may specify multiple -U arguments within one call of avrdude. * Note that some numerical values refer to fuses containing undefined bits (set to '1' here). Depending on the target device these fuse bits will be read either as '0' or '1'. Verification errors will occur if the values are read back with undefined bits set to '0'. Everything is fine if the values read from the device are either the same as programmed, or the following values (undefined set to '0'):Extended: 0x07. |