# Description:

Drives up to 1 watt of LEDs per color. Perfect for a single 3 watt star RGB LED or up to 20 5mm RGB LED. Supports common anode or common cathode, changed via jumpers. The versitility of the driver supports a range of LED(s) and input voltage combinations.

Can be used for mood lamps, case mods, battery operated lanterns, display lights, and more.

# Features:

- Drives up to 2 watt of LEDs per color.

- Dial and Pushbutton control of speed and functions.

- Auto fade w/ speed adjust via dial

-Manual color select via dial

-Dimming via dial

- Remembers user settings when turned on/off

- Pushbutton changes function, quick press changes between auto and manual, push and hold enables dimming

- Compatible with common anode, common cathode

or single color LED(s)

Parts shown are for Common Anode LED with 5 volt Input

# Specification:

Full kit with 3 watt RGB LED

Board with Custom Resistors

Input Voltage

5v*

Input Voltage

5v-12v**

Input Current	< 1 Amp

Input Current

< 1 Amp

Output Red

~350ma

Output Red

up to 2 watts

Output Green	~350ma

Output Green

up to 2 watts

Output Blue

~350ma

Output Blue

up to 2 watts

Connectors	0.1” Pitch

Connectors

0.1” Pitch

Dimension

1.5”x1.5“x0.5”

Dimension

1.5”x1.5“x0.5”

*5v available if the 7805 is not installed

**7-12 volts availible with simple modifications see Fig 4a & 4b

For Questions or Comments E-Mail to Support@ChromationSystems.com

Standard Layout

see Fig. 3c for Header Info

Header 1

- Blue LED

- Green LED

- Red LED

- Positive/Negative Header 2

- Positive, +5 v

- Negative, 0v

- Switch

- Potetiometer Center Tap R1 - 5.1 ohm, 1w

R2 - 16z ohm, 1w

R3 - 16 ohm, 1w R4 - 500 ohm, 1/4w R5 - 500 ohm, 1/4w R6 - 500 ohm, 1/4w R7 - 10kohm

T1 - Transistor, 2.5watt T2 - Transistor, 2.5watt T3 - Transistor, 2.5watt C1 - 0.1 uf disc capacitor Jumper B1 to B2  Jumper A1 to A2

Also:

2 Pin Header 8-Pin Socket

12F675 PIC Microcontroller

Header 1

Header 2

Tools Required:

- Soldering Iron

- Solder

- Needle Nose Pliars

Optional:

  - Volt Meter

  - Electrical Tape

  - Hot Glue Gun

For Common Cathode LED

J1 jumped to POS1 J2 jumped to NEG2

Header 1, pin 4 goes to LED cathode

T1-T3 are placed opposite of Top-Side outline

For Common Anode LED

J1 jumped to NEG1 J2 jumped to POS2

Header 1, pin 4 goes to LED anode T1-T3 are placed in as pictured

Common Anode, 5 volt Input	Common Cathode, 5 volt Input

Fig. 2a	Fig. 2b

When using 5 volt as the input voltage, the 7805 is not used. So a jumper must be ran from Pin 1 to Pin 3, and the Solder pad for pin 2 needs to be filled in with solder. see Fig 2a & 2b

Pictured is the layout for the full kit with 3 watt RGB LED, with 5 volt input.

Fig. 3c

Header 1

- Blue LED

- Green LED

- Red LED

- LED Positive/Negative Header 2

- Positive, +5 v

- Negative, 0v

- Switch

- Potetiometer Center Tap

*careful not to plug the into the wrong header

Fig. 3a	Fig. 3b

*full size schematics availble on the website.

###### Common Anode, 7+ volt Input	Common Cathode, 7+ volt Input

Fig. 4a	Fig. 4b