# How To Connect NQ HMI To Omron PLC Using Hostlink Protocol

###### Introduction This article describes the initial connection between a NQ HMI and a PLC using Hostlink protocol. For the example below a NQ3-TQ000B and a CP1L PLC will be used. Cables required for a good connection are: NQ-CN222 or NQ-CN521 to connect to CP1W CIF01 or directly to CJ1M PLC. For pin-outs of these cables look in the appendix of the introduction manual (V07-EN-0*). NQ-CN221 is also available for connection to the mini-peripheral connector for CJ1. NQ: - Create a new application. - Go to COM tab or nodes folder and add Host link protocol and appropiate PLC (standard communication settings ,9600,1,7,2,E). - After clicking add you will see following been created in Nodes folder: Important: Default node number is 1 - Create a PLC tag in the tagdatabase, example CIO 100. - Open screen 1 and place a Numeric Data Entry on the screen. - Assign the created PLC tag to the Numeric data entry. - Download application AND firmware into the NQ PLC: - Create a new application - Got to PLC Settings, Serial Port and select Mode Host link and standard settings. - Go to unit number and change to 1: - Download changed settings to PLC. Connection: After connecting the correct cable to the communication will be established which will result in a steady Blue RUN led on the NQ. The Numeric datadisplay will show the value of CIO100 of the PLC. Attached you will find a small sample application for NQ3 and Hostlink setting. In this program also CIO100 is used to display the value. Pressing the + or - buttons will increase / decrease the value of CIO100. On a CP1L PLC you will directly see the result in as outputs will be switched on or off. Also an empty PLC program (CP1L empty) for CP1L-L PLC is added for correct settings. Downloading both NQ3 and PLC sample program to your hardware AND using the correct cable should result in a working data exchange between PLC and NQ3.