==ORCADNET.DOC===========Info on TANGO.CF File============Version 1.1==

OrCAD/SDT users  have been stymied in their use of Tango-PCB and Tango-

PCB PLUS  since Tango  was updated  to Version  2.   At  that  time  an

autoplacement feature was added to Tango-PCB PLUS and thus both PCB and

PCB PLUS  began to  care about  what was  on the  second  line  of  the

component section of the Tango-format net list.  Heretofore, the second

line was ignored.  The component section of the net list looks thus:

[

refdes

package

type

value (for certain components)

(blank line)

(blank line)

]

OrCAD/SDT's net  list generator  has  routinely  placed  the  component

"type"  on   the  "package"  line  should  no  package  information  be

available.   The "type"  is then  specified a second time on the "type"

line.

For the  sake of  clarity, terminology by ACCEL and OrCAD translates as

follows:

Tango/ACCEL              OrCAD

-----------              -----

RefDes                   Reference

Package (aka Pattern)    Module Name

Type                     Part Name (aka Part Value)

With the  introduction of  OrCAD/SDT IV, the net list formats have been

externalized to the program and are interpreted at run time.  Therefore

it was possible to edit the TANGO.CF net list configuration file to fix

the aforementioned problem.

Using the new TANGO.CF file (a note within the file indicates it is now

called Version 1.1 to distinguish it from the original file), OrCAD/SDT

4.04 users  can generate  a net  list which will leave the Package line

blank if  the components  on the schematic were not edited with Package

information.

However, users  of Tango-PCB PLUS wishing to take full advantage of its

autoplacement facility  should read  on.   OrCAD/SDT will permit you to

edit components  on the  sheet and  place Package  information with the

component.    By  properly  configuring  the  net  list  generator,  as

described below,  this Package  information will  then be placed on the

Package line  of the net list.  Then, after being loaded into Tango-PCB

PLUS, the components may be autoplaced, saving a lot of time.

To edit  the components  in OrCAD/SDT Draft, run the Edit Part command,

select one  of the  available "Part  Fields" 1 through 8, (we recommend

Page 1

==ORCADNET.DOC===========Info on TANGO.CF File============Version 1.1==

using the  "1st Part  Field"), select "Name" on the next menu, and then

type in  the name  of the  Package (ie,  DIP16) that you wish to use in

Tango-PCB PLUS for that component.  All parts of a multi-part component

should have identical Package names.  (See page 77 of the SDT Reference

Guide for information on Edit Part, and page 34 for more information on

"Key fields.")

Now, prior to executing the net list select "Configure Schematic Design

Tools."   Page down  several times  until you  find the section "Create

Netlist."   Under that  section are two parameters:  In the "Part Value

Combine" field,  key in a "V"  --  In the "Module Value Combine" field,

key in  the number  of the Part Field you selected to place the Package

information.   For example,  if using  Edit Part you placed the Package

name in  the 1st  Part Field, key in "1."  If you selected the 8th Part

Field, key  in an  "8." (See  page 42  of the  SDT Reference  Guide for

information on this configuration procedure).

The net  list generator  will extract  the data  in the  Part Field and

combine it  on the  "Module Name"  line of  the net list, which is just

where Tango wants to see it.

Page 2