==RTEPLUS.DOC============== Tango-Route PLUS ==============Version 2.10==

INTRODUCTION

This file  contains the  latest information  regarding  Version  2.10  of

Tango-Route PLUS.   It  is to be used in conjunction with the Tango-Route

PLUS Reference Manual.

Note that  the .PCB  file formats have been enhanced from Version 1.XX to

include support  for polygons,  additional signal  layers (in  PCB PLUS),

editable power  and ground  planes, autoplacement  (in PCB  PLUS), and an

imbedded net  list for Route.  Version 2.XX of Tango-PCB will read either

Version 1.XX  or Version  2.XX files and produce a 2.XX file as a result.

Tango-Route only reads a Version 2.XX file.  Thus, Tango-PCB Version 2.XX

is a prerequisite to Tango-Route Version 2.XX.

INSTALLATION INSTRUCTIONS

ACCEL is standardizing on an automatic installation program, INSTALL,

which uses compressed files on the distribution diskettes.  The Tango-

Route PLUS Reference Manual contains complete instructions for using this

utility.  The minimum free disk space required for a full installation is

1 MB.

NEW FEATURES and ENHANCEMENTS

The following  features have  been added  or improved  in  this  release.

Please consult  the Release  Notes for  a detailed  description of  these

enhancements.

SMART NETS  - The router now adds Nets information to lines and vias that

it places.   The router also maintains the individual Net Names

for any  connections that it doesn't route.  You will no longer

have all of the connections grouped together in a no-routes net

when you  call up  the board  in PCB.  Each connection line now

has the  appropriate Net Name (Nets ID of a blue line will show

the net name).

BINARY FILE  SUPPORT -  The router now supports binary files created with

Tango-PCB version  2.10.   The router will automatically detect

if the  file is  binary or ASCII, and it will write out the PCB

file in  the same  format as  the original  file.  Binary files

will be approximately half the size of ASCII files.

BUG FIXES

1.   Routing into  Polygons and  Keepouts -  We fixed a problem that

would sometimes  result in  traces being  routed through polygon and

keepout areas.

2.   Duplicating PWR  and GND  Traces -  We fixed a problem that was

causing the router to duplicate pre-routes on the PWR and GND planes

each time the board was routed.

Page 1

==RTEPLUS.DOC============== Tango-Route PLUS ==============Version 2.10==

KNOWN PROBLEMS AND OTHER CONSIDERATIONS

1.   The MAIN  MENU selection  available on the menu which is pulled

down during  routing clears all memory allocated to the routing

process and returns control to the main menu in preparation for

a new  route.   If the program was invoked with a .SET filename

on  the   command  line,  the  previous  setup  information  is

preserved, allowing  the designer  to readily change parameters

and begin another route.

2.   Tango-Route PLUS  accepts connections  made with  polygon fills

for pre-routes  in all  cases except  as part of the enumerated

Wide-Line nets.  Polygon fills used as pre-routes for Wide-Line

nets are currently ignored.

3.   Tango-Route PLUS  will route your pre-routed connections if the

endpoints of the line segments are not concentric and/or if the

endpoints are  not connected  exactly to  pad  centers.    This

problem occurs  most often  when the pre-routes are done on a 1

mil grid.   Make  sure all  of the  endpoints are centered over

other endpoints  or pad  centers to  avoid  this  problem.    A

similar situation  exists for T routes.  Route will often route

pre-routed T routes that are not made up of 3 endpoints

4.   Tango-Route PLUS  may try to route to pads that have the center

point covered  by a  keepout.  To avoid this problem, make sure

that the  entire pad  that you want protected is covered by the

keepout.

Page 2