;=======================================================================================

;                   Speed Controller With Full Bridge Output

;=======================================================================================

;                    Bruce Abbott bhabbott@paradise.net.nz

;

; for Microchip PIC12F629 or PIC12F675

;

; Features:

;

;     - Bi-directional control of brushed DC motor

;     - Neutral throttle detected at 1.5mS or 1.37mS

;     - LVC proportional to initial battery voltage

PROCESSOR PIC12F675

INCLUDE   <P12F675.inc>

radix     dec

;#DEFINE SIMULATE	; for debugging only!

#DEFINE SLOW		; rate-limited throttle response

#define VERSION "=BRIDGE===V0.1=="

; =====================================================================================

; Configuration is:

;   Master Clear pin is disabled (used as input)

;   Code Protection is OFF

;   Watchdog Timer is ON

;   Oscillator is Internal RC

__config  _MCLRE_OFF & _CP_OFF & _WDT_ON & _INTRC_OSC_NOCLKOUT

; GPIO register bits

RevLoBit        EQU       0   ; GP0, pin 7. Turns on motor reverse FET

BattBit         EQU       1   ; GP1, pin 6. Battery status input

ForLoBit        EQU       2   ; GP2, pin 5. Turns on motor forward FET

ServoBit        EQU       3   ; GP3, pin 4. Servo pulse input

RevHiBit        EQU       4   ; GP3, pin 3. Turns on high-side reverse FET

ForHiBit        EQU       5   ; GP4, pin 2. Turns on high-side forward FET

; TRIS register value

TrisBits        EQU       (1<<ServoBit)|(1<<BattBit)

; Bits for H bridge activation

ON_FORWARD      EQU (1<<ForLoBit)|(1<<ForHiBit)

OFF_FORWARD     EQU (1<<ForHiBit)

ON_STOP         EQU 0

OFF_STOP        EQU 0

ON_REVERSE      EQU (1<<RevLoBit)|(1<<RevHiBit)

OFF_REVERSE     EQU (1<<RevHiBit)

; Bits to be set with the OPTION instruction

;   No wake up

;   No weak pullups

;   Timer 0 source internal

;   Which edge is don't care

;   Prescaler to watchdog, set to give 16*17 = 272ms timeout

OptionBits      EQU       B'11011100'

; =====================================================================================

; Algorithm constants

;

ArmPulsesReqd   EQU       25   ; number of good servo pulses required to allow starting

PWMLOOPS        EQU       42   ; number of steps in Motor PWM cycle

PRECHARGE	EQU	-112   ; pre-charge servocount for roll over at 0.9mS

;======================================================================================

; Macro to create offsets for general purpose registers in RAM

;

_addr           SET 32    ; general purpose registers start here

BYTE            MACRO     _name

_name           EQU       _addr

_addr           SET       _addr+1

ENDM

; =====================================================================================

; General Purpose Register Definitions.

;

BYTE       Flags           ; Various boolean flags

BYTE       PwmCount        ; Counts down to zero during PWM output

BYTE       PwmWidth        ; PWM loop counter

BYTE       MotorON         ; GPIO output bits for motor ON

BYTE       MotorOFF        ; GPIO output bits for motor OFF

BYTE       ServoCount      ; servo pulse count 0-255 * 8uS

BYTE       HiCycles        ; counts PWM cycles during servo pulse

BYTE       PrevCount       ; Previous servo pulse width

BYTE       Index           ; Index into table of PWM widths (0-75)

BYTE       ArmPulseCount   ; Arming pulses required (counts down)

BYTE       PwmPeriod       ; Number of 8uS loops in one PWM cycle

BYTE	   Volts	   ; battery voltage

BYTE       temp		   ; general purpose variable

; Bits in Flags

BattLow         EQU       0	; low battery detected

PriorServoBit   EQU       1	; servo pulse was high

BadPulseBit     EQU       2	; servo pulse was bad

Stop1		EQU	  3	; last arm pulse was in table 1 stop area

Stop2		EQU	  4	; last arm pulse was in table 2 stop area

UseTable1	EQU	  5	; using throttle table 1

UseTable2	EQU	  6	; using throttle table 2

; ==================================================================================

; Macro for generating short time delays

;

NO_OP           MACRO   count

NO_OP_COUNT     SET     count

WHILE   NO_OP_COUNT>1

goto	$+1		; 2 clocks

NO_OP_COUNT     SET     NO_OP_COUNT-2

ENDW

IF	NO_OP_COUNT

nop			; 1 clock

ENDIF

ENDM

; ==================================================================================

; Macro to start PWM Cycle

;

; Takes 4 cycles to execute

;

StartPWM        MACRO

movf	 MotorON,W

movwf    GPIO

movf     PwmWidth,W

movwf    PwmCount

ENDM

; =================================================================================

; Macro to keep up with the servo pulse duration

; The code in this Macro uses 2 cycles.

;

DoServoCount    MACRO

btfsc     GPIO, ServoBit

incf      ServoCount, F

ENDM

;==================================================================================

;                                  RESET

;==================================================================================

;

Coldstart:      ORG       0

goto	Start

org	  0x8

dt        VERSION

;=================================================================================

;                          Throttle Curve Table

;================================================================================

; Returns with PWM width and Reverse Flag (bit 7) in W.

; Takes 5 CPU cycles to execute (including call and retlw).

;

; There are two tables. The second table is accessed by adding 75 to W.

;

; Table1:- Has equal forward and backward range, for stick with neutral at 1.5mS

; Table2:- More forward than reverse range, for stick with neutral at 1.37mS

;

Table:	    addwf   PCL,F      ; computed goto, W = 0  to 150

; Table 1

retlw   128+43     ; 0  reverse max at 1.2mS

retlw   128+42     ; 1

retlw   128+42     ; 2

retlw   128+41     ; 3

retlw   128+41     ; 4

retlw   128+40     ; 5

retlw   128+40     ; 6

retlw   128+39     ; 7

retlw   128+38     ; 8

retlw   128+37     ; 9

retlw   128+36     ;10

retlw   128+35     ;11

retlw   128+34     ;12

retlw   128+33     ;13

retlw   128+32     ;14

retlw   128+31     ;15

retlw   128+30     ;16

retlw   128+29     ;17

retlw   128+28     ;18    reverse

retlw   128+26     ;19

retlw   128+24     ;20

retlw   128+22     ;21

retlw   128+20     ;22

retlw   128+18     ;23

retlw   128+16     ;24

retlw   128+14     ;25

retlw   128+12     ;26

retlw   128+10     ;27

retlw    128+8     ;28

retlw    128+6     ;29

retlw    128+4     ;30

retlw    128+2     ;31

retlw        0     ;32 }

retlw        0     ;33 }

retlw        0     ;34 }

retlw        0     ;35 }

retlw        0     ;36 }

retlw        0     ;37 }  STOP at 1.5mS

retlw        0     ;38 }

retlw        0     ;39 }

retlw        0     ;40 }

retlw        0     ;41 }

retlw        0     ;42 }

retlw        2     ;43

retlw        4     ;44

retlw        6     ;45

retlw        8     ;46

retlw       10     ;47

retlw       12     ;48

retlw       14     ;49

retlw       16     ;50

retlw       18     ;51

retlw       20     ;52

retlw       22     ;53

retlw       24     ;54

retlw       26     ;55  forward

retlw       27     ;56

retlw       28     ;57

retlw       29     ;58

retlw       30     ;59

retlw       31     ;60

retlw       32     ;61

retlw       33     ;62

retlw       34     ;63

retlw       35     ;64

retlw       36     ;65

retlw       37     ;66

retlw       38     ;67

retlw       39     ;68

retlw       40     ;69

retlw       40     ;70

retlw       41     ;71

retlw       41     ;72

retlw       42     ;73

retlw       42     ;74

retlw       43     ;75 forward max at 1.8mS

; Table 2

retlw    128+30    ; 0  reverse max at 1.2mS

retlw    128+29    ; 1

retlw    128+28    ; 2

retlw    128+27    ; 3

retlw    128+26    ; 4

retlw    128+25    ; 5

retlw    128+24    ; 6

retlw    128+22    ; 7

retlw    128+20    ; 8

retlw    128+18    ; 9   reverse

retlw    128+16    ;10

retlw    128+14    ;11

retlw    128+12    ;12

retlw    128+8     ;13

retlw    128+6     ;14

retlw    128+4     ;15

retlw        0     ;16 }

retlw        0     ;17 }

retlw        0     ;18 }

retlw        0     ;19 }

retlw        0     ;20 }

retlw        0     ;21 }  STOP at 1.37mS

retlw        0     ;22 }

retlw        0     ;23 }

retlw        0     ;24 }

retlw        0     ;25 }

retlw        0     ;26 }

retlw        2     ;27

retlw        4     ;28

retlw        6     ;29

retlw        8     ;30

retlw       10     ;31

retlw       12     ;32    forward

retlw       14     ;33

retlw       16     ;34

retlw       18     ;35

retlw       20     ;36

retlw       22     ;37

retlw       23     ;38

retlw       24     ;39

retlw       25     ;40

retlw       26     ;41

retlw       27     ;42

retlw       28     ;43

retlw       29     ;44

retlw       30     ;45

retlw       30     ;46

retlw       31     ;47

retlw       31     ;48

retlw       32     ;49

retlw       32     ;50

retlw       33     ;51

retlw       33     ;52

retlw       34     ;53

retlw       34     ;54

retlw       35     ;55

retlw       35     ;56

retlw       36     ;57

retlw       36     ;58

retlw       37     ;59

retlw       37     ;60

retlw       38     ;61

retlw       38     ;62

retlw       39     ;63

retlw       39     ;64

retlw       40     ;65

retlw       40     ;66

retlw       40     ;67

retlw       41     ;68

retlw       41     ;69

retlw       41     ;70

retlw       42     ;71

retlw       42     ;72

retlw       42     ;73

retlw       42     ;74

retlw       43     ;75 forward max at 1.8mS

;----------------------------------------------------------------------------------

;                   Battery Voltage Cutoff Table

;----------------------------------------------------------------------------------

CutoffTable:

addwf	PCL,F	; BATTVOLTS   CUTOFF    BATTERY TYPE

retlw	3	; 0   4.0V

retlw	3	; 1   4.5V

retlw	3	; 2   5.0V

retlw	3	; 3   5.5V

retlw	3	; 4   6.0V     5.5V   6V SLA

retlw	3	; 5   6.5V     5.5V   6V SLA

retlw	4	; 6   7.0V     6.0V   7.2V Nicad (6 cells)

retlw	4	; 7   7.5V     6.0V   7.2V Nicad (6 cells)

retlw	4	; 8   8.0V     6.0V   7.2V Nicad (6 cells)

retlw	6	; 9   8.5V     7.0V   8.4V Nicad (7 cells)

retlw	6	; 10  9.0V     7.0V   8.4V Nicad (7 cells)

retlw	6	; 11  9.5V     7.0V   8.4V Nicad (7 cells)

retlw	8	; 12 10.0V     8.0V   9.6V Nicad (8 cells)

retlw	8	; 13 10.5V     8.0V   9.6V Nicad (8 cells)

retlw	8	; 14 11.0V     8.0V   9.6V Nicad (8 cells)

retlw	14	; 15 11.5V    11.0V   12V SLA

;==================================================================================

;                         Measure Battery Voltage

;==================================================================================

;

BattVolts:	movlw	15

movwf	Volts		; start comparison at maximum Volts

bv_next:	bsf	STATUS,RP0

movf	Volts,w

movwf	VRCON		; set Vref divider to Volts

bsf	VRCON,VREN	; enable Vref

bcf	STATUS,RP0

NO_OP	4		; wait 5uS

btfss	CMCON,COUT	; Battery voltage higher than VRef?

goto	bv_done		; yes, done

decfsz	Volts		; no, try lower Vref

goto	bv_done

bv_done:	return

;==================================================================================

;                        Measure servo pulse width

;==================================================================================

;

Measure:	movlw	PRECHARGE

movwf	ServoCount		; start servocount at -0.9mS

bcf	Flags,BadPulseBit

Measure1: 	clrwdt

btfss	GPIO,ServoBit		; wait for start of pulse

goto	Measure1

Measure2:  	clrwdt				;(1)>

nop				;(1)> measuring first 0.9mS

incf	ServoCount,F		;(1)>

skpnz				;(2)> done 0.9mS ?

goto	Measure3		;(+1)

btfsc	GPIO,ServoBit		;(1)> pulse finished early ?

goto	Measure2		;(2)>

MeasureError:	clrf	ServoCount		;     bad if pulse < 0.9mS

goto	MeasureDone

Measure3:	clrwdt				;(1)]

nop				;(1)] measuring to end of pulse

incf	ServoCount,F		;(1)]

skpnz				;(2)] bad if pulse > 2.9mS

goto	MeasureError		;

btfsc	GPIO,ServoBit		;(1)] done if end of pulse

goto	Measure3		;(2)]

MeasureDone:	movf	ServoCount,w

return

; ================================================================================

; PWM pulse subroutine.  On entry, the motor may be on or off. If the motor

; is off, it will never be turned on.  However, it will be turned off when/if

; the PwmCount register reaches zero.

;

; takes 350 CPU cycles.

;

; Minimum ON time is 27uS

; 2uS inside StartPWM + 18uS + 7uS inside DoPwmPulse.

;

; Minimum OFF time is 13uS

; 7uS inside DoPwmPulse + 4uS + 2uS inside StartPWM.

;

;

;      2,2        18        7           336 cycles          7    4    2,2

;        ____________________                                           _______

;  _____/                    \\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\___________/

;        ON                     OFF at (PwmCount-1) * 8uS              ON

;    (startPWM)             |<--------- DoPwmPulse -------->|       (startPWM)

;

;

DoPwmPulse:     movlw     PWMLOOPS              ;(1) set PWM period counter

movwf     PwmPeriod             ;(1)

movf      MotorOFF,W            ;(1) get GPIO value for motor off

pwmloop:        decf      PwmCount,F            ;(1)

skpnz                           ;(1) IF end of PWM high time

movwf     GPIO                  ;(1) THEN turn off motor

IFDEF	SIMULATE

nop

nop

ELSE

btfsc     CMCON,COUT            ;(2)

;goto      BattLoShutdown        ; quit if battery low

ENDIF

decfsz    PwmPeriod,F           ;(2)

goto      pwmloop               ;(+1)

retlw     0                     ;(2) done

;------------------------------------------------------------------------------------

;                           END OF SUBROUTINES

;------------------------------------------------------------------------------------

ORG       256

; Retrieve and set the factory-programmed oscillator calibration value.

Start:		call	0x3ff

bsf	STATUS,RP0	; Register bank 1

movwf   OSCCAL          ; set oscillator frequency

movlw	b'00000010'

movwf	ANSEL		; Analog I/O on GP1

bcf	STATUS,RP0      ; register bank 0

movlw	b'00000100'

movwf	CMCON		; Comparator in- to GP1, in+ to VREF

;

; Move the prescaler from tmr0 to the watchdog, without accidental resets.

;

clrwdt

clrf      TMR0

movlw     OptionBits | 7

option

clrwdt

movlw     OptionBits

option

clrwdt

; Initialise GPIO register

;

clrf    GPIO		; all outputs will be low

movlw   TrisBits

tris    GPIO		; set I/O directions

InitDelay:	clrf	PwmPeriod

InitDelay1:	clrf	temp

InitDelay2:	clrwdt

NO_OP	4		; wait 0.5s for peripherals to stabilize

decfsz	temp

goto	InitDelay2

decfsz	PwmPeriod

goto	InitDelay1

; set battery cutoff voltage

call	BattVolts	; get battery voltage

movf	Volts,w

call	CutoffTable	; calculate cutoff voltage

bsf	STATUS,RP0      ; register bank 0

movwf	VRCON		; set VRef voltage for cutoff

bsf	VRCON,VREN	; enable VRef

bcf	STATUS,RP0      ; register bank 0

clrf      Flags		; clear memory

Rearm:		clrwdt

IFNDEF	SIMULATE

btfsc   CMCON,COUT	; wait for good battery voltage

;goto    Rearm

ENDIF

;

; Wait for a whole servo pulse before looking for arm pulses

;

RaWaitOn:	clrwdt

btfss   GPIO, ServoBit

goto    RaWaitOn

RaWaitOff: 	btfsc   GPIO, ServoBit

goto    RaWaitOff

;

; wait until throttle is in neutral position

;

RaStartCount:   movlw   ArmPulsesReqd

movwf   ArmPulseCount	; set required number of good pulses

bcf	Flags,Stop1

bcf	Flags,Stop2

RaNextPulse:	call	Measure		; measure servo pulse width

skpnz			; bad pulse ?

goto	RaStartCount

movwf	PrevCount	; remember servo pulse

movwf	Index

movlw	38

subwf	Index,F

skpc			; minimum = 1.2mS

clrf	Index

movlw	75

subwf	Index,W		; maximum = 1.8mS (index range: 0 to 75)

skpnc

subwf	Index,F

btfsc	Flags,UseTable2	; using throttle table 2 ?

goto	RaTable2	; yes, check for table 2 STOP

clrf	PCLATH

movf	Index,w

call	Table

xorlw	0		; Table1 STOP ?

skpnz

goto	RaStop1

btfsc	Flags,UseTable1	; using thottle table 1 ?

goto	RaStartCount	; no, stick not at neutral

RaTable2:	clrf	PCLATH

movf	Index,w

addlw	76

call	Table

xorlw	0		; Table 2 STOP ?

skpz

goto	RaStartCount	; no, stick not at neutral

RaStop2:	bsf	Flags,Stop2	; now in Table 2 STOP region

btfss	Flags,Stop1	; was last pulse in Table 1 STOP region ?

goto	RaCount		; no, continue

bcf	Flags,Stop1

goto	RaStartCount	; yes, restart

RaStop1:	bsf	Flags,Stop1	; now in table 1 STOP region

btfss	Flags,Stop2	; was last pulse in Table 2 STOP region ?

goto	RaCount		; no, continue

bcf	Flags,Stop2

goto	RaStartCount	; yes, restart

RaCount:	decfsz	ArmPulseCount,F	; Do we have enough arm pulses?

goto	RaNextPulse	; Not yet.

;

; Set throttle table to use

;

btfsc	Flags,Stop1

bsf	Flags,UseTable1

btfsc	Flags,Stop2

bsf	Flags,UseTable2

;

; Play a 2KHz 'beep' sound for 1/8 second, to indicate successful arming.

; (uses the motor as a speaker!)

;

PlayBeep:

clrf      PwmCount

beep1:          movlw	  ON_FORWARD

movwf	  GPIO		       ; motor on

movlw     5

movwf     PwmPeriod

beep2:          clrwdt                         ; for 22uS

decfsz    PwmPeriod,F

goto      beep2

movlw	  OFF_FORWARD

movwf	  GPIO		       ; motor off

movlw     128

movwf     PwmPeriod

beep3:          clrwdt

decfsz    PwmPeriod,F

goto      beep3                ; for 0.5mS

decfsz    PwmCount,F

goto      beep1                ; 256 cycles

; wait for end of next servo pulse

servo_hi:       btfss     GPIO,ServoBit

goto      servo_hi

servo_lo:       btfsc     GPIO,ServoBit

goto      servo_lo

; ready to go!

clrf      MotorON              ; Motor FETs OFF

clrf      MotorOFF             ; Direction FETs OFF

clrf      PwmWidth             ; PWM=0

movlw     PRECHARGE

movwf     ServoCount           ; precharge ServoCount

goto	  MainPwmEntry

; ==================================================================================

; The main PWM loop

;

; Two timing loops have to be simultaneously maintained:-

;

; 1/ Measure servo pulse width every 8uS. DoServoCount takes 2 CPU

;    cycles, thus 6 CPU cycles are available between each instance.

;

; 2/ StartPWM + 18 cycles + DoPwmPulse + 4 cycles etc. This works

;    out to 372 CPU cycles, creating a PWM frequency of 2.7Khz.

;

; The watchdog timer is cleared only AFTER we have received a good servo

; pulse. Loss of signal will cause the PIC to reset itself!

;

;

MainPwmLoop:    DoServoCount                   ;(2) <--- }

movlw     PWMLOOPS             ;(1)      }

movwf     PwmPeriod            ;(1)      }

movf      MotorOFF,W           ;(1)      }

pwmpulseloop:   decf      PwmCount,F           ;(1)      }

skpnz                          ;(1)      } <=== Complete PWM cycle.

movwf     GPIO                 ;(1)      }      Also measure servo

DoServoCount                   ;(2) <--- }      pulse width.

decfsz    PwmPeriod,F          ;(2)      }

goto      pwmpulseloop         ;(+1)     }   (takes 342 CPU cycles)

IFDEF	SIMULATE

nop

nop

;ELSE

;btfsc     CMCON,COUT           ;(2)      }

;goto      BattLoShutdown       ; quit if battery low

;ENDIF

NO_OP     2                    ;(2)

DoServoCount                   ;(2) <---

MainPwmEntry:   StartPWM                       ;(4) ====> start 1st PWM cycle

NO_OP     2                    ;(2)

DoServoCount                   ;(2) <---

btfsc     GPIO, ServoBit       ;(2)

goto      ServoIsHigh          ;(+1) Is servo pulse High ?

btfsc     Flags, PriorServoBit ;(2)

goto      EndOfPulse           ;(+1) end of servo pulse ?

movlw	  PRECHARGE	       ;(1)

movwf     ServoCount           ;(1) pre-charge servocount

DoServoCount                   ;(2) <---

clrf      HiCycles             ;(1)

NO_OP     3                    ;(1)

goto      MainPwmLoop          ;(2)

ServoIsHigh:    bsf       Flags, PriorServoBit ;(1) servo pulse is High!

NO_OP	  2		       ;(2)

DoServoCount                   ;(2) <---

incf      HiCycles,F           ;(1) Count PWM cycles while servo is high

skpnz                          ;(1)

decf      HiCycles,F           ;(1) avoid HiCycles overflow

NO_OP     1                    ;(1)

goto      MainPwmLoop          ;(2)

; ============================================================================

; End of Servo Pulse. We don't have to measure the servo pulse width now, but

; we still have to maintain the PWM cycle:-

;

;   StartPWM + 18 clocks + DoPwmPulse + 4 clocks + StartPWM etc.

;

;

; gets here 9 clocks after start of 1st PWM

EndOfPulse:     bcf       Flags,BadPulseBit     ;(1) assume servo pulse is OK

movlw     2                     ;(1)

subwf     HiCycles,W            ;(1) short pulse ? (less than 3 PWM's)

skpc                            ;(1)

bsf       Flags,BadPulseBit     ;(1)

movlw     7                     ;(1)

subwf     HiCycles,W            ;(1) long pulse ? (more than 6 PWM's)

skpnc                           ;(1)

bsf       Flags,BadPulseBit     ;(1)

call      DoPwmPulse            ;(350) <=== Complete 1st PWM cycle

movlw     163                   ;(1)

subwf     ServoCount,W          ;(1) servo pulse width in-range ?

skpnc                           ;(1) ( 0.9mS < pulse < 2.2mS)

bsf       Flags,BadPulseBit     ;(1)

StartPWM                        ;(4) ====> Start 2nd PWM cycle

btfsc     Flags,BadPulseBit     ;(2)  ignore bad pulses

goto      BadPulse              ;(+1)

IFDEF  SLOW ; change-rate limited throttle

movf	  ServoCount,w		;(1) compare throttle to last position

subwf	  PrevCount,w		;(1)

skpc	  			;(2) going up ?

goto	  ThrottleUp		;(+1)

skpnz				;(2) going down ?

goto	  NoChange  		;(+1)

ThrottleDown:	addlw	  -2			;(1)

skpnc				;(1)

addwf	  ServoCount		;(1) servocount = prevcount - 2

NO_OP	  3			;(3)

goto	  Slowed		;(2)

NoChange: 	NO_OP	  5			;(5)

goto	  Slowed		;(2)

ThrottleUp:	addlw	  2			;(1)

skpc				;(1)

addwf	  ServoCount		;(1) servocount = prevcount + 2

NO_OP	  6			;(6)

Slowed:		movf	  ServoCount,w		;(1)

movwf	  Index			;(1)

ELSE	; mean-averaged throttle

movf      ServoCount,W          ;(1)

movwf     Index                 ;(1) Index = ServoCount (0-163)

movf      PrevCount,W           ;(1)

addwf     Index,F               ;(1)

rrf       Index,F               ;(1) Index = Average(ServoCount+PrevCount)

NO_OP	  11			;(11)

ENDIF

call      DoPwmPulse            ;(350) <=== Complete 2nd PWM cycle

NO_OP	  4			;(4)

StartPWM			;(4) ====> Start 3rd PWM cycle

movlw     38                    ;(1)

subwf     Index,F               ;(1)

skpc                            ;(1) minimum = 1.2mS

clrf      Index                 ;(1)

movlw     75                    ;(1)

subwf     Index,W               ;(1) maximum = 1.8mS

skpnc				;(1)

subwf	  Index,F		;(1)

movlw	  76			;(1)

btfsc	  Flags,UseTable2	;(1) using second throttle table ?

addwf	  Index			;(1)

clrf	  PCLATH		;(1) required for simulation only!

NO_OP	  6			;(6)

call      DoPwmPulse            ;(350) <=== Complete 3rd PWM cycle

NO_OP     4                     ;(4)

StartPWM                        ;(4) ====> Start 4th PWM cycle

movf      Index,W               ;(1)

call      Table                 ;(5) do table lookup

movwf     PwmWidth              ;(1)

btfsc     PwmWidth,7            ;(2) reverse flag set ?

goto      reversing             ;(+1)

tstf      PwmWidth              ;(1) if PWM Width = 0

skpnz                           ;(2) then stop

goto      stopping              ;(+1)

movlw     ON_FORWARD            ;(1) MotorON set for Forward

movwf     MotorON               ;(1)

movlw     OFF_FORWARD           ;(1) store new MotorOFF in temp (must

movwf     temp                  ;(1) complete this PWM with old MotorOFF)

goto      oldpwm                ;(2)

reversing:      bcf       PwmWidth,7            ;(1) remove reverse flag from PwmWidth

movlw     ON_REVERSE            ;(1) MotorON set for Reverse

movwf     MotorON               ;(1)

movlw     OFF_REVERSE           ;(1) store new MotorOFF for Reverse

movwf     temp                  ;(1)

NO_OP     1                     ;(1)

goto      oldpwm                ;(2)

stopping:       movlw     ON_STOP               ;(1) MotorON set for Stop

movwf     MotorON               ;(1)

movlw     OFF_STOP              ;(1) Store new MotorOFF for Stop

movwf     temp                  ;(1)

NO_OP     1                     ;(1)

oldpwm:         call      DoPwmPulse            ;(350) <=== Complete 4th PWM cycle

movf      temp,W                ;(1)

xorwf     MotorOFF,F            ;(1) are new MotorOFF bits same as old ?

skpz                            ;(1) if not, we are changing direction

movwf     GPIO                  ;(1) so turn motor OFF now!

StartPWM                        ;(4) ====> Start next PWM cycle

movf      temp,W                ;(1)

movwf     MotorOFF              ;(1) set new Motor_OFF

movf      ServoCount,W          ;(1)

movwf     PrevCount             ;(1) remember this servo pulse

NO_OP     2                     ;(2)

clrwdt				;(1) Good pulse, so clear watchdog

BadPulse:       bcf       Flags,BadPulseBit     ;(1)

movlw     PRECHARGE             ;(1)

movwf     ServoCount            ;(1) precharge ServoCount

bcf       Flags,PriorServoBit   ;(1) get ready for next servo pulse

clrf      HiCycles              ;(1)               ''

NO_OP     6                     ;(6)

call      DoPwmPulse            ;(350) <=== Complete PWM cycle

NO_OP     2                     ;(2)

goto      MainPwmEntry          ;(2)

;

; Low battery - shutdown and wait for arming.

;

BattLoShutdown: movf      MotorOFF,W

movwf     GPIO                  ; Stop the motor

goto      Rearm			; restart

org	0x3ff

RETLW	0x80		; OSCCAL value programmed at factory

END