http://hyperphysics.phy-astr.gsu.edu/hbase/electric/indsol.html#c1

http://hyperphysics.phy-astr.gsu.edu/hbase/magnetic/indcur.html

https://www.changpuak.ch/electronics/calc_21.php

https://hamwaves.com/inductance/en/index.html#input

Mas 4mm coil

D = 0.75mm

L = 31mm+108mm+a két beforrasztott maradék

8 menet

QOIL™ — https://hamwaves.com/qoil/ — v20181217

Coil design 2020-01-29 19:13

INPUT

mean diameter of the coil    D = 4.5 mm

number of turns              N = 8

length of the coil           ℓ = 10 mm

wire or tubing diameter      d = 0.75 mm

design frequency             f = 400 MHz

The (plating) material is hard-drawn copper.

INTERMEDIATE RESULTS

winding pitch                p = 1.25 mm

physical conductor length    ℓ_w_phys = 113.5 mm

effective pitch angle        ψ = 5.31°

RESULTS

Effective equivalent circuit

effective series inductance @ design frequency          L_eff_s = 0.100 μH

effective series reactance @ design frequency           X_eff_s = 251.9 Ω

effective series AC resistance @ design frequency       R_eff_s = 0.423 Ω

effective unloaded quality factor @ design frequency    Q_eff = 595

Lumped circuit equivalent

f-independent series inductance; geometrical formula    L_s = 0.091 μH

series AC resistance @ design frequency                 R_s = 0.348 Ω

parallel stray capacitance @ design frequency           C_p = 0.2 pF

Self-resonant frequency                                   f_res = 1165.714 MHz