The XY Pen Plotter to HPGL translation

Version 0.10 4 feb 2007

By Emile van der Laan

www.emsign.nl

Introduction	1

Capture file	1

Software Design	2

RS232 interface	2

Plotter commands	2

Hardware Design	2

# Introduction

I have made this project for the Philips PM3355 scoop as it only has a XY output,

And the version that I have does not a printer output.
But it has a simple XY Plotter output.

It will convert a XY Input and Pen Up/Down to a HPGL file.

This file you can include in a Word document, this is not standard any more. But if you install the KB196506 then you still can do it on 32 Bits Windows versions.

I have chosen this format as it is one of the vector formats that have not so much overhead on initialization.

As the AVR Tiny13 only has 1 KB of flash Rom and then every byte counts.

# Capture file

The Baud Rate is about 9600 baud.

Set up the terminal program in that way that it will capture all the communication.

Send a character this will send the HPGL initialization commands back to you

Then start the XY plot function.

When the Plot function is ready send a character this will stop the sending of new coordinates.

And close the capture file.

# Insert in word

After you have the capture file you can insert the picture From file in word.

Then a pop up box will appear en then you will need to select the HP Graphics Language.

# Software Design

The software will send the internalization commands they are simple HPGL commands.

The Sample rate will need to be set in software.

## RS232 interface

The Baud Rate is 9600 baud, but you can have it faster if you want you will need to change in the PcRs232.S file.

I have used a software Uart, This is both because then I do not need a RS232 level converter, And that the Avr Tiny does not have a Uart.

The Receive code is in the PcRs232.S file but is not used.

The “Problem” is that depending on the internal RC oscillator this can be a bit off.

Then you will need to change the divider that is in the PCRs232.S file.

And this is also why having this in a production unit is not really recommended.

## Plotter commands

Some explanation you can find at the HPGL Wiki location.

IN;  			Initialize, start a plotting job

IP;			Set the initial point (origin), in this case the default 0,0

DI;

SP1;			Select pen 1

SC0,1024,0, 1024;	Set the scale so the page is 1024,1024 in both X and Y directions

PU0,0;			Lift the pen and move to 0,0

PD931,0;		Put down the pen and move to the following locations draw a box

PD931,931;

PD0,931;

PD0,0;

# Hardware Design

The X and Y inputs will have a sensibility from 0 Volt to 1 Volt

The Pen Up/Down has a Pullup resistor so that it can handle an opencollector Output form the Scoop.

I have added a 1K resistor in the X and Y line, This is just to protect my Tiny.

The schematic:

# Programming the Tiny 13

I have included a batch file to program the Tiny 13 avrdudeprog.cmd.

And the communicationpoort9600.ht as this will start hyperterminal.

Have fun !