WHISTLE DETECTOR

Description

Although very simple, this circuit provides a reliable way of controlling an external device by means of whistling. The PIC12C509 microcontroller was used after several failed attempts at implementing a reliable circuit using analog techniques (tone decoders, bandpass filter, etc ).

Using a microcontroller to decode the amplified frequency signal (saturated - providing a square wave output!)  proved a very reliable solution.

The firmware can be changed to fine tune the system for individual needs. There are two registers (MIN_TIME, MAX_TIME ) which provide a low and high frequency cut-off point. Another register (COUNTER2) controls the length of the required activating whistle. The signal output is active high and remains high for about 500msec when a whistle is detected. To assemble the code, just copy and paste the code shown below into your pic development environment.

PIC12C509 FIRMWARE CODE

;********************************************************************

;*	Whistle Detector

;*	by Keith Wilson

;********************************************************************

list p=12c509,r=hex		; microcontroller & base

;----- Register Definitions  ---------------------------------------

W		EQU     H'0000'

F		EQU     H'0001'

INDF		EQU     H'0000'

TMR0		EQU     H'0001'

PCL		EQU     H'0002'

STATUS	EQU     H'0003'

FSR		EQU     H'0004'

OSCCAL	EQU     H'0005'

GPIO		EQU     H'0006'

;----- STATUS Bits --------------------------------------------------------

PA2		EQU     H'0007'

PA1		EQU     H'0006'

PA0		EQU     H'0005'

NOT_TO	EQU     H'0004'

NOT_PD	EQU     H'0003'

Z		EQU     H'0002'

DC		EQU     H'0001'

C		EQU     H'0000'

;----- OPTION Bits --------------------------------------------------------

T0CS		EQU     H'0005'

T0SE		EQU     H'0004'

PSA		EQU     H'0003'

PS2		EQU     H'0002'

PS1		EQU     H'0001'

PS0		EQU     H'0000'

;----- Whistle Detector Definitions ---------------------------------------

WHIOUT	EQU     0               ; whistle output - signal

WHILED	EQU     1               ; whistle output - led

WHIIN		EQU     3               ; whistle input signal

COUNTER1	EQU     0x07            ;counter for signal low time

COUNTER2	EQU     0x08            ;counter for signal repeatition before being recognized

MIN_TIME	EQU     0x09            ;highest frequency capture range limit - period time

MAX_TIME	EQU     0x0a            ;lowest frequency capature range limit - period time

;********************************************************************

;Program Start

;********************************************************************

main	org 	0x00			;start of program

movlw	0x0C

tris	GPIO			;set inputs/outputs

bsf	GPIO,WHILED		;turn green led on

movlw	0x23			;MIN_TIME, high freq. cut-off equiv. ca 0.25usec(0x23)

movwf	MIN_TIME

movlw	0x80			;MAX_TIME, low freq. cit-off equiv. ca 0.50usec(0x43)

movwf	MAX_TIME

start	movlw	0x80			;numberof times the signal should be repeated

movwf	COUNTER2

loop1	btfsc	GPIO,WHIIN		;is signal low? - skip if true

goto	loop1			;no - wait until signal goes high

bcf	GPIO,WHILED		;turn green led off

movlw	0x00			;reset counter1

movwf	COUNTER1

loop2	btfsc	GPIO,WHIIN		;is signal still low? - skip if true

goto	sigtes			;signal is high - see if freq in capture range

incfsz	COUNTER1,F		;increment counter 1 - is zero(overroll) then skip

goto	loop2			;wait until signal goes high

goto	start			;signal out off capture range - start again!

sigtes	movf	MIN_TIME,W		;compare signal low time for high frequency

subwf	COUNTER1,W		;if COUNTER1>MIN_TIME then skip

bc	$+2			;signal is within lower capture range

goto	start			;signal not in capture range - start again!

movf	COUNTER1,W		;compare signal low time for low frequency

subwf	MAX_TIME,W		;if MAX_TIME>COUNTER1 then skip

bc	$+2			;signal is within higher capture range

goto	start			;signal not in capture range - start again!

decfsz	COUNTER2,F		;signal must be repeated COUNTER2 times before recognition!

goto	loop1			;test for another low condition

whidet	bsf	GPIO,WHIOUT		;whistle detected! - turn signal on

call	delay			;wait approx 0.5 sec. before turning signal off

call	delay			;insert more calls to delay to extend signal output

bcf 	GPIO,WHIOUT		;turn signal off

goto	start

;*****************************************************************************************

; DELAY - routine causes a small delay of approx 200 msec at 4MHz (counter1=counter2=0xff)

;*****************************************************************************************

delay	movlw	0x80

movwf	COUNTER2

loop5	movlw	0xff

movwf	COUNTER1

loop4	decfsz	COUNTER1,F

goto	loop4

decfsz	COUNTER2,F

goto	loop5

retlw	0

end